Design Overview

My design uses a "Streaming Line Buffer" architecture. Instead of waiting for the entire image to load which can take a long time and lot of memory this design processes the image pixel by-pixel as it flows in from the camera or image file. To perform the time Convolution which needs data from the pixels above in mode 2 I used two Line Buffers. These act like a short-term memory, holding exactly two rows of the image so the processor can see and read  the neighbours of the current pixel.

Design Evolution & Learning Process

My final architecture was chosen after analysing different approaches:

- Initial approach in my first try I tried to process pixels directly without buffering. I missed the alternate inputs which made it impossible and it was wrong approach as there is difference in clk speed of processor and producer to perform spatial filtering like edge detection and inverting of pixels.

- My solution is based on the learning from Boot Camp reference material, I implemented the Line Buffer approach that help me to avoid the skipping of data

- I observed a slight delay (latency) of one clock cycle between the Producer and the Processor. Reason is this delay is intentional and necessary. It comes from the Pipeline Registers and the Valid/Ready Handshake the benefit is it adds a tiny delay it ensures the data is stable and synchronized before the math calculations happen. This prevents "glitches" in the image and metastability.

Advantage is speed. In a software approach a processor has to read each pixel one by one and do the math and save it takes many clock cycles for every single dot. This design does all the filiping, multiplication and addition in parallel using a pipeline. This means it produces one finished pixel every single clock cycle making it fast enough for real time processing.

Finally, the design is safer because of the synchronization logic I learned from the boot camp materials. I noticed a small delay of one clock cycle between the producer and the processor, but this is actually a good thing. This delay comes from the handshake signals that ensure data is stable before it gets processed. This prevents glitches and metastability.